ECA0801 – ANALOG AND DIGITAL COMMUNICATION

EXPERIMENT 2 – Characteristics Study of Side Band Power and Frequencies in AM Using MATLAB

Objective 1 : To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and modulating signal parameters

Software Required : MATLAB

Program Code :

clc;

clear;

close all;

Am = 1;

fm = 500;

Ac = 5;

fc = 5000;

fs = 20*fc;

t = 0:1/fs:4/fm;

m = Am*sin(2*pi*fm*t);

c = Ac*sin(2*pi*fc*t);

mu = Am/Ac;

am = Ac*(1+mu*sin(2*pi*fm*t)).*sin(2*pi*fc*t);

dem = abs(hilbert(am));

dem = dem-mean(dem);

N = length(am);

f = linspace(-fs/2,fs/2,N);

AM_FFT = abs(fftshift(fft(am)))/N;

USB = fc+fm;

LSB = fc-fm;

Pc = Ac^2/2;

Psb = (mu^2*Pc)/4;

figure('Name','AM Modulation','NumberTitle','off');

subplot(3,2,1);

plot(t,m);

title('Message');

grid on;

subplot(3,2,2);

plot(t,c);

title('Carrier');

grid on;

subplot(3,2,3);

plot(t,am);

title('AM Signal');

grid on;

subplot(3,2,4);

plot(t,dem);

title('Demodulated');

grid on;

subplot(3,2,5);

plot(f,AM_FFT);

xlim([fc-3*fm fc+3*fm]);

title('Sidebands');

grid on;

subplot(3,2,6);

bar([Pc Psb Psb]);

set(gca,'XTickLabel',{'Carrier','USB','LSB'});

title('Power');

grid on;

fprintf('mu = %.2f\n',mu);

fprintf('USB = %d Hz\n',USB);

fprintf('LSB = %d Hz\n',LSB);

fprintf('Carrier Power = %.2f W\n',Pc);

fprintf('Each Sideband Power = %.3f W\n',Psb);

OUTPUT :

Objective 2 : To determine and verify the upper sideband (USB) and lower sideband (LSB) frequencies of the AM signal using frequency spectrum analysis in MATLAB.

Software Reduired : MATLAB

Program code :

clc;

clear;

close all;

fc = 50;

fm = 5;

Ac = 1;

m = 1;

fs = 2000;

t = 0:1/fs:1;

am = Ac*(1+m*sin(2*pi*fm*t)).*sin(2*pi*fc*t);

N = length(am);

f = (-N/2:N/2-1)*fs/N;

F = abs(fftshift(fft(am)))/N;

USB = fc+fm;

LSB = fc-fm;

figure;

subplot(2,1,1);

plot(t,am);

title('AM Signal');

xlabel('Time (s)');

grid on;

subplot(2,1,2);

plot(f,F);

xlim([30 70]);

title('AM Frequency Spectrum');

xlabel('Frequency (Hz)');

grid on;

fprintf('USB = %d Hz\n',USB);

fprintf('LSB = %d Hz\n',LSB);

OUTPUT :

Objective 3 : To calculate the carrier power, sideband power, and total transmitted power of the AM signal for a given modulation index using MATLAB.

Software Required : MATLAB

Program code :

clc;

clear;

close all;

Pc = 0.5;

m = 1;

Pusb = (m^2*Pc)/4;

Plsb = (m^2*Pc)/4;

Pside = Pusb+Plsb;

Pt = Pc+Pside;

bar([Pc Pusb Plsb Pt]);

set(gca,'XTickLabel',{'Carrier','USB','LSB','Total'});

ylabel('Power (W)');

title('AM Power Distribution');

grid on;

fprintf('Carrier Power = %.3f W\n',Pc);

fprintf('USB Power = %.3f W\n',Pusb);

fprintf('LSB Power = %.3f W\n',Plsb);

fprintf('Total Sideband Power = %.3f W\n',Pside);

fprintf('Total Transmitted Power = %.3f W\n',Pt);

OUTPUT :

Objective 4 : To analyze the effect of varying the modulation index on sideband power distribution and overall transmission efficiency through MATLAB simulation.

Software Required : MATLAB

Program Code :

clc;

clear;

close all;

Pc = 0.5;

m = [0.2 0.5 0.8 1.0];

Pusb = (m.^2*Pc)/4;

Plsb = Pusb;

Pside = Pusb+Plsb;

Pt = Pc+Pside;

eff = (Pside./Pt)*100;

figure;

subplot(2,1,1);

plot(m,Pside,'o-');

title('Sideband Power vs Modulation Index');

xlabel('Modulation Index');

ylabel('Total Sideband Power (W)');

grid on;

subplot(2,1,2);

plot(m,eff,'o-');

title('Transmission Efficiency vs Modulation Index');

xlabel('Modulation Index');

ylabel('Efficiency (%)');

grid on;

disp('Modulation Index:');

disp(m);

disp('Total Sideband Power (W):');

disp(Pside);

disp('Transmission Efficiency (%):');

disp(eff);

OUTPUT :

SAMPLE LAB EXAM QUESTIONS

- Generate an AM signal in MATLAB using a carrier frequency of 10kHz, modulating frequency of 1kHz, carrier amplitude of 10V, and modulation index m=0.5. Plot the AM waveform and frequency spectrum. Determine and verify the LSB, carrier and USB frequencies.

Program Code :

clc;

clear;

close all;

Ac = 10;

fc = 10000;

fm = 1000;

m = 0.5;

fs = 20*fc;

t = 0:1/fs:4/fm;

am = Ac*(1+m*sin(2*pi*fm*t)).*sin(2*pi*fc*t);

N = length(am);

f = (-N/2:N/2-1)*fs/N;

F = abs(fftshift(fft(am)))/N;

LSB = fc-fm;

USB = fc+fm;

figure;

subplot(2,1,1);

plot(t,am);

title('AM Waveform');

xlabel('Time (s)');

ylabel('Amplitude (V)');

grid on;

subplot(2,1,2);

plot(f,F);

xlim([fc-3*fm fc+3*fm]);

title('AM Frequency Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

fprintf('LSB = %d Hz\n',LSB);

fprintf('Carrier = %d Hz\n',fc);

fprintf('USB = %d Hz\n',USB);

OUTPUT :

- For an AM signal with carrier amplitude 10V, load resistance 1Ω, and modulation index m=0.5, calculate the carrier power, USB power, LSB power and total transmitted power using MATLAB. Determine the transmission efficiency and compare the theoretical and simulated values.

Program code :

clc;

clear;

close all;

Ac = 10;

R = 1;

m = 0.5;

Pc = Ac^2/(2*R);

Pusb = (m^2*Pc)/4;

Plsb = (m^2*Pc)/4;

Pside = Pusb+Plsb;

Pt = Pc+Pside;

eff = (Pside/Pt)*100;

figure;

bar([Pc Pusb Plsb Pt]);

set(gca,'XTickLabel',{'Carrier','USB','LSB','Total'});

ylabel('Power (W)');

title('AM Power Distribution');

grid on;

fprintf('Carrier Power = %.3f W\n',Pc);

fprintf('USB Power = %.3f W\n',Pusb);

fprintf('LSB Power = %.3f W\n',Plsb);

fprintf('Total Sideband Power = %.3f W\n',Pside);

fprintf('Total Transmitted Power = %.3f W\n',Pt);

fprintf('Transmission Efficiency = %.2f %%\n',eff);

OUTPUT :